Prepared by W6JIM

STANDARD QSO Protocol

PROTOCOL-1

(their call) DE (your call)

- GM/GA/GE ES TNX FER (CALL/RPRT)		Key-Words:

- UR RST 599

- QTH FRESNO, CA  	    x2		     	                      RST-QTH-NAME

- NAME JIM

- OK HW?  AR

(their call) DE (your call) K

PROTOCOL-2

(their call) DE (your call)

- OK (John) FB ES TNX FER (INFO/RPRT)   		Notes:

- RIG IC 7300 ES PWR 50W

- ANT DIPOLE UP 45 FT

- WX Rain ES TEMP 50F

- OK HW?  AR

(their call) DE (your call) K

PROTOCOL-3

(their call) DE (your call)

- OK (John) SOLID CPY 					Notes:

- AGE 55 YRS

- BEEN HAM FER 25 YRS

- MY KEY J38

- OK HW?  AR

(their call) DE (your call) K

PROTOCOL-4 (Ending)

(their call) DE (your call)			           	Notes:

- OK (John) TNX FER FB QSO

- ES HP CUAGN 73  AR

(their call) DE (your call) TU  SK

(Send closing “Dits” as described on next page)

Additional Info

- QRL? - Before calling CQ. Call QRL? at least twice to ensure the frequency is not in use

- Calling CQ - A typical CQ call is in a 2 x 2 format: CQ CQ DE (your call) (your call) K

  - Wait then repeat if necessary, again 2 x 2. No need to call lengthy CQs

  - Stations most typically respond with (your call) DE (their call) (their call) K

  - When a station answers your CQ begin protocol P1 using the TNX FER CALL option (Standard)

- Answering a CQ - Answer a CQ by sending (their call) DE (your call) (your call) K

  - The calling station will go through protocol-1 then hand it back to you

  - Now you start protocol-1 using the TNX FER RPRT option (Standard)

- Ending - other station starts the ending sequence:

  - Come back after the SK like you normally would for a K

  - Answer any open questions, say anything else you want to share, then initiate P4-Ending

  - After sending your SK send 2-dits.  The other station will send 2-dits back & your QSO is done!

- Ending - you start the ending sequence:

  - Wait for the other station to finish their turn after you SK

  - Send 2-dits after the other station sends theirs and your QSO is done!

- Repeat important information twice during Standard QSOs.  Your choice during Quick QSOs

  - RST, QTH, and Name and anything else that you feel is important (like club numbers)

- BK - When you get handed the QSO with a BK you acknowledge by starting with a BK.  BK is not a pro-sign, so there is a little space between the B and the K.

  - Example:

    - (other station) OK JIM, UR RIG AGN? BK

    - (W6JIM)  BK RIG HR IS TEN TEC EAGLE, TEN TEC EAGLE BK

    - (other station) BK OK JIM FB ON UR TEN TEC EAGLE….

- R R - stands for Roger Roger.

  - It is customary (not required) to send R R before you start your portion of the exchange

  - Only do this if you fully understood the other stations message

  - Example:  R R  (their call) DE (your call) FB CPY JOHN ES TNX FER NICE RPRT

- BT, Comma and Period - Use these to separate topics.  BT is also used a pause.

- Standard (their Call) DE (your call) exchanges

  - In a Standard QSO, each turn starts and ends with (their Call) DE (your call)

- For extended Rag-Chews

  - Write down some info to share in case you draw a blank

  - You may start with the standard exchange and roll into the abbreviated BK exchange

  - But don’t forget to identify your station with a standard exchange every 10 minutes

- The examples in this reference sheet are just that, examples

  - Feel free to tailor your content as you become more comfortable and hear different versions

  - Name vs OP, NR QTH vs Actual QTH, BT vs Pause, CUAGN vs CUL, CPY vs CPI, etc.…

  - Please edit this document to make it work for you!